# CAPL Simulation Script – Generic Predefined Rules (Authoritative)

## 1. File-Level Structural Rules (Mandatory)

- Exactly one CAPL file = one simulation node

  - The CAPL node represents one logical ECU or one simulator ECU.

  - Do NOT mix multiple ECU responsibilities unless explicitly stated in requirements.

- Order of sections is fixed

- includes

- variables

- on start

- timers (logical usage)

- on message

- on timer

- on key

Any deviation = incorrect script.

- DBC is the single source of truth

  - All message names, signal names, scaling, enums must match DBC exactly

  - Never invent messages, signals, or units.

## 2. variables Block – When and How to Use (Strict Rules)

### Use variables ONLY for:

- State persistence

  - ECU internal state

  - Previous signal values (edge detection)

  - Alive counters

  - Mode flags (Ignition ON/OFF, Hazard ACTIVE, etc.)

- Timers

- msTimer t_alive;

- msTimer t_periodic;

- Counters and flags

- int aliveCounter = 0;

- int hazardActive = 0;

### DO NOT put in variables:

- Temporary variables used only inside one event

- Message objects (declare locally in on timer or on message)

- Test verdict logic

### Hard Rule

If a value must survive beyond one event → variables
If not → local variable

## 3. on start Block – Purpose and Constraints

### Use on start ONLY for:

- Initial ECU state

- ignitionStatus = OFF;

- hazardActive = 0;

- aliveCounter = 0;

- Starting periodic timers

- setTimer(t_alive, 100);

- Default signal initialization (only if ECU transmits at startup)

### DO NOT in on start:

- Send event-triggered messages

- Simulate faults

- React to test cases

### Hard Rule

on start = power-up behavior only
No runtime logic allowed

## 4. on message Block – Event-Driven ECU Behavior

### Use on message ONLY when:

- ECU reacts to a received CAN message

- Requirement says:

  - “When X is received…”

  - “Upon change of…”

  - “If signal transitions from A to B…”

### Correct Usage Pattern

on message BCM_Status

{

if (this.Door_FL != prevDoorFL)

{

prevDoorFL = this.Door_FL;

// Trigger action

}

}

### Mandatory Rules

- Always use this.SignalName

- Always perform edge detection when reacting to changes

- Never transmit periodically from on message

- Never use timers here

### DO NOT

- Use delays

- Use setTimer() unless explicitly required by requirement

- Combine multiple unrelated requirements

## 5. on timer Block – Periodic and Delayed Behavior

### Use on timer ONLY for:

- Periodic transmissions

  - Alive messages

  - Cyclic status frames

  - Heartbeat counters

- Delayed reactions

  - Timeout supervision

  - Deferred actions defined in requirements

### Correct Pattern

on timer t_alive

{

message Gateway_Alive msg;

msg.Counter = aliveCounter++;

output(msg);

setTimer(t_alive, 100);

}

### Hard Rules

- Every periodic requirement → exactly one timer

- Timer must re-arm itself

- Timing must match requirement exactly

### DO NOT

- React to incoming messages here

- Handle key presses

- Change ECU modes unless requirement explicitly states timeout logic

## 6. on key Block – Test Control Only

### Use on key ONLY for:

- Manual stimulation during testing

- Forcing ECU states

- Debug overrides

### Allowed Keys

on key 'h'  // Hazard toggle

on key 'i'  // Ignition ON/OFF

### Strict Separation Rule

on key is NOT ECU behavior
It is test bench control only

### DO NOT

- Encode production logic

- Replace message-based logic with keys

- Depend on keys for requirement compliance

## 7. Mapping ECU Requirements → CAPL Blocks (Non-Optional)

| Requirement Type | CAPL Block |
| --- | --- |
| Periodic transmit | on timer |
| React to received CAN | on message |
| Power-up behavior | on start |
| Alive / heartbeat | on timer |
| Event-triggered transmit | on message |
| Manual stimulation | on key |

## 8. Alive Counter Rules (Mandatory Pattern)

- Counter increments modulo defined bit length

- Sent only from timer

- Reset only on power cycle

aliveCounter = (aliveCounter + 1) % 16;

## 9. Message Transmission Rules (Absolute)

- Every transmitted message:

  - Must be defined in DBC

  - Must use a local message object

- output(msg) only

- No hardcoded CAN IDs

## 10. Forbidden Practices (Immediate Rejection)

- Mixing test logic and ECU logic

- Using while(1) loops

- Sending periodic messages from on message

- Ignoring edge detection

- Using magic numbers instead of requirement timing

- Using keys to satisfy requirements